# CPE 301: Altera FPGA Tutorial for 5CSEMA5F31C6N

# Spring 2019

This tutorial will guide you through the following steps:

- Creating a new project

- Adding Source Code

- Assigning Package Pins to Signals

- Uploading the Code to the FPGA

# Creating a New Project

- Open Quartus II from the Start Menu / Desktop

- File -> New Project Wizard

- Create your own project folder on the Desktop.

  - Name it something unique.

  - Make sure it is on the local desktop, not on your cloud drive.

  - Explicitly place in on the C: drive : (C:\Users\<MY_USERNAME>\Desktop)

- Choose the folder you created and name the project gate2.

- Click Next

- We don’t have any design files to add right now, so click Next

- Select the correct device:

  - Family: Cyclone V

  - Device: 5CSEMA5F31C6N

- Click Next

- We won’t be using these advanced features, so click next.

- Review the project settings, and click finish.

This is the general display of the new project. We’ve completed setting up our new project.

# Adding new source files

- Let’s add a new source code file to the project: File -> New -> Verilog HDL File

A blank file with the name Verilog1.v appears.

- Now save it as gate2.v: File -> Save as

- Copy this example code into the source file:

module gate2 (in_a, in_b, out);

input wire in_a; // declare input variable in_a

input wire in_b; // declare input variable in_a

output wire [5:0] out; // declare out variable out

assign out[5] = in_a&in_b; // AND operation

assign out[4] = ~(in_a&in_b); // NAND operation

assign out[3] = in_a | in_b; // OR operation

assign out[2] = ~(in_a | in_b); // NOR operation

assign out[1] = in_a ^ in_b; // XOR operation

assign out[0] = ~(in_a ^ in_b); // NXOR operation

endmodule

Your screen should then look like this:

- Right-click the source file in the file hierarchy and set the file as the top level entity. (You may need to select “Files” in the project navigator)

- Double-check that you selected the correct device:

Tool bar ->Assignments ->Devices..: You should see Cyclone V, 5CSEMAF31C6.

You can also go to the Boards menu and select the DE1-SoC at the bottom of the list.

- Start compiling the code:

Tool Bar -> Processing -> Start Compilation

Make sure the module name matches the file name:

# Assigning Pins to Signals

- Assign the pins to the signals.

a) Tool bar ->Assignments -> Pins

b) a new window,“Pin Planner”, should appear as below:

Note: if you don’t see the bottom window, go to View ->All Pins List from the Pin Planner window.

Here are the pin assignments you’ll need as provided by Quartus:

LED Pins:

Slide Switch Pins:

Push Button Pins:

7 Segment Pins:

- Now we will assign the pins as follows:

- Close the Pin Planner window.

# Uploading Code to the Board

- Compile the code again by going to Processing ->Start Compilation. The Message window should look like this when compilation is successful.

- Go to Tools ->Programmer, a new window appears:

- Check if the Hardware Setup section is USB-Blaster. If not, install the USB driver as follow:

a. Go to device manager, select Other Devices and select the USB-Blaster -> Update Driver

Software -> Browse my computer for driver software.

b. Point to the folder: C:\altera\91sp2\quartus\drivers\usb-blaster where C:\altera is the

installation folder of Quartus.

c. Click Next -> Done.

d. If the driver is successfully installed, you should be able to select USB-Blaster in

Hardware Setup.

- Select the Correct Device

- Add the programming file, by clicking “Add File”

- Add the “gates.sof” file

- Remove any file listings which may have been added by default, they will not have 5CSMA5F31 in the Device column.

- Click Start

We have programmed the FPGA, verify that the code we programmed is working as expected.